# EGB240 1B Assessment

- Sawtooth Falling | 2400fmin | 5200fmax | 25finc

- Week 5 Lecture Notes

Conversion between binary and hex

Pulse Width Modulation is a method of driving an analogue system from a digital quantity

Low pass filtering can be applied to eliminate the harmonics t and above f(PWM)

- PWM must be kept at 50% for this assignment

- Period of PWM. T(PWM) = Top x T

- Duty cycle = output compare register /Top

Microcontroller I/O

- DDRx, PINx, PORTx

- Reading from PINx. Writing as an output from PORTx

- Polling vs Interrupt design;

Polling is an infinite loop continuously checking if an event has occurred and taking appropriate action.

Disadvantage> Waste processor time

Interrupt driven is when an event occurs, the main program execution pauses and control passes to the interrupt service routine.

Button Debouncing

- Bounce can happen when opening or closing a switch

- Debouncing can be done via hardware or software solution

- Sample the signal then consider stable value

Week 5 Lectorial

Demo 0

- Load, build and download skeleton code to Teensy

- Source(src) == main C code

Demo 1

- Not pressed 5V. Pressed 0V

- LED output >> PortD

- PortF 7,6,5,4 => 0 inputs

- PortD 7,6,5,4 => 1 outputs

Demo 2

- Period of 1sec. B6 can be configured for OC1B or OC4B is the Jout for this assessment

- Timer control TCCR1A

Normal mode. 3rd mode >> fast PWM

5,4 bits >>

waveform control >> 3,2,1,0

Mode 14 or 15 >> Use mode 15

We want 1s for T(PWM)

Pre scaler of 256

1000

Clear >> compare match

Set >> Top

F(pwm) = 1/T

Top = T/ 1.048   x  65535